COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES FOR DEVELOPMENT AND CONSTRUCTION OF EXHIBITIONS, DISPLAYS AND

GALLERY REFURBISHMENT

March 1997

(Revised June 2001,

March 2014)

Louise Bacon

Head Collections Conservation & Care

HORNIMAN MUSEUM & GARDENS

COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES

FOR DEVELOPMENT AND CONSTRUCTION OF EXHIBITIONS, DISPLAYS AND

GALLERY REFURBISHMENT

CONTENTS

Introduction to Standards

Glossary

No.1 Handling Museum Objects

No.2 Gallery Refurbishment

No.3 Specifications for Design of Showcases

No.4 Case Construction Materials

No.5 Case Dressing Materials

No.6 Open Displays

No.7 Mounts and Fixings

No.8 Testing Materials & Off-gassing

Appendices I – III: Horniman Museum Codes of practice

HORNIMAN MUSEUM & GARDENS

COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES FOR DEVELOPMENT AND CONSTRUCTION OF EXHIBITIONS, DISPLAYS AND

GALLERY REFURBISHMENT

Introduction to Standards

The standards and guidelines have been drawn up to act as a guide for Museum staff, Exhibition Designers and Contractors and must be taken into consideration when planning the development, construction and execution of Exhibitions, Displays or Gallery Refurbishments whether TEMPORARY or PERMANENT or subject to budget.

The main objective of this document is to cover most of the CONSERVATION and ENVIRONMENTAL issues at an early stage in the development of a Project.

As with all policies the Standards and Guidelines in this document will be reviewed periodically and updated.

The Standards and Guidelines form part of one of the Museum's primary objectives, that of Collections, which states “we will safeguard and develop the collections so that they can be made as accessible as possible now and into the future”

Horniman Museum & Gardens Corporate Plan 2014-2016.

This is supported through the Collections Conservation & Care policy. 2002

HORNIMAN MUSEUM & GARDENS

COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES FOR DEVELOPMENT AND CONSTRUCTION OF EXHIBITIONS, DISPLAYS AND

GALLERY REFURBISHMENT

Glossary

Throughout the document the following short terms have been used to signify:

Object(s):         		Any specimen, artefact, object registered by the Museum

Display:			Exhibition, display or Gallery Refurbishment

Conservation :   	Head of Collections Conservation & Care

Exhibitions:        	Head of Exhibitions & Design

Facilities:             	Facilities Manager

Collections:       	 	Collections Manager

SCC:                  	Study Collections Centre

Engineer:         		Relates mainly to Mechanical and Electrical Works

M&E:               		Mechanical & Electrical

For Further Information on the Standards please contact:

Head of Collections Conservation & Care

The Horniman Museum & Gardens

100 London Road

Forest Hill

London SE23 3PQ

Tel No: 020 8291 8682

Email: www.enquiries@horniman.ac.uk

Fax No. 020 8291 5506

HORNIMAN MUSEUM & GARDENS

COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES FOR DEVELOPMENTAND CONSTRUCTION OF EXHIBITIONS, DISPLAYS AND

GALLERY REFURBISHMENT

No.1 Handling Museum Objects

Standard

All actions must minimise the movement and handling of objects.

No person should handle museum specimens unless they have received suitable object handling training provided by Conservation.

Health & Safety Risk Assessments may be required before any object is moved.

No smoking, eating or drinking except in designated areas.

Guidelines

Among the many causes for the deterioration of museum objects are light, temperature, humidity, vibration, dust and noxious chemicals. Environmental hazards are insects, rodents, fungus, biotic organisms and by far the greatest cause of destruction, people.

All objects must be handled by approved and trained Museum staff or their approved representatives. Approval and training is dependent on Conservation. (see Appendix III.)

Storage and laying out space for objects to be included in a display is limited at both the SCC and the Horniman Museum and must be taken into consideration in any planning programme. Spaces should be booked through Collections Management or Conservation

Identification of the contents of boxes or items under covers should be possible without the removal of the object for examination.

Viewing of objects at the SCC, or the Museum, as well as movement between the two sites necessitates the following steps:

Retrieving object	(handling & movement)

Packing		(handling & movement)

Transportation	(movement & vibration

whether across the room or by

vehicle)

No.1 Handling Museum Objects contd.

Unpacking			 (handling & movement)

Arranging object for viewing (handling & movement)

Re-packing			(handling & movement)

Transportation		(movement & vibration)

Re-locating object		(handling & movement)

Consequently a planned programme of handling 'sessions’ for whole Project i.e. from inception through progressing the design, to conservation treatment and installation must be presented and approved by Exhibitions, Conservation and Collections and any other Section in the museum who is relevant to the process. In this way different sessions can be programmed to run concurrently or in logical sequence. The programme must be carefully planned so as to minimise the number of times an object is moved and handled.

During installation, consideration must be given to where objects can be held in a  'safe area' while mounting and fixing takes place.

'Safe areas' must be designated and form part of the planned programme of handling 'sessions' for example during the installation of the object(s) on display or during viewing sessions. A 'safe area' must primarily provide protection against damage, dust and theft. A ‘safe area’ needs to be approved by Conservation. Facilities would need to be consulted on security issues. (see also No.6 Open Displays)

	At least 24 hours must be allowed for some objects that have been transported between the SCC and the Horniman Museum to acclimatise to new spaces before unpacking e.g. ivory.

Objects must be carried in a supporting container at all times unless size dictates otherwise. Conservation to advise and train approved personnel as required. Health & Safety manual handling training may also be required.

No.1 Handling Museum Objects contd.

	When not being viewed objects must be protected from dust and covered with acid free tissue paper, Tyvek (spun bonded polyethylene) or other covering as specified by Conservation. ‘Object Underneath’ labels are available from Conservation and should be used.

	Floor loading capacities and the allowance of suitable access to move objects in and out of the Gallery must be considered. Health & Safety Risk Assessments may also be required.

	Objects should neither be subjected to undue vibration or shock nor extremes of temperature, humidity and light. (Conservation to advise)

HORNIMAN MUSEUM & GARDENS

COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES FOR DEVELOPMENT AND

CONSTRUCTION OF EXHIBITONS, DISPLAYS AND

GALLERY   REFURBISHMENT

No.2 Gallery Refurbishment

Standard

Any adaptations to the fabric, running of trunking, ductwork or any other fixtures or changes must take into consideration the status of the Grade II * Listed Building.

Guidelines

	Conservation / Facilities may require temperature/humidity sensors to be installed to connect with the Museum's Building Management System, or stand-alone electronic monitors as appropriate.

	Facilities/Exhibitions may require security/alarm installations.  Care must be taken to avoid installing systems with conflicting radio frequencies to the Hanwell environmental monitoring system (434.075 mHz)

Any materials such as carpet, insulation, floor adhesives etc. must be tested and approved by Conservation (see No. 8 Testing Materials and Off gassing)

	Any building works, such as plastering, painting or laying screed must be allowed time to dry out. (see No. 8 Testing Materials and Off Gassing)

- Any changes to climate control provision must be carried out in consultation with Conservation, Facilities, Exhibitions and M & E Engineer. Good air circulation within the galleries must be maintained at all times.

	Floor loading capacities and the allowance of suitable access to move objects in and out of the galleries must be considered.	               (see Appendix II)

HORNIMAN MUSEUM & GARDENS

COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES FOR DEVELOPMENT AND   CONSTRUCTION OF EXHIBITIONS, DISPLAYS AND

GALLERY REFURBISHMENT

No.3 Specification for Design of Showcases.

Standards

Museum showcases need to provide a sturdy, secure environment as free as possible from dust, pests, pollution and heat.  They must also be easy to open so that the displays can be maintained and modified with ease.

Guidelines

The specifications given below are an initial statement and should be regarded as a basis for discussion. Designers and Contractors should work closely with Conservation, Facilities and Exhibition staff  to detail showcase specification.

It is not intended to give a detailed list of materials that can or cannot be used in a Museum context; the working practices described below are for guidance only.

- Carcass	 	Fully enclosed with separate ventilated light			box.

All interior joints must be airtight.

Removable panels to be provided to any

enclosed space around or beneath showcases to allow for cleaning.

Some showcases may require tight

control of their air exchange rate (to be

advised by Conservation).

- Access		Preferably via hinged doors or glass panels,

or sash windows, failing that pull and slide glass panels.

Use of removable glass panels is to be

avoided if possible.

One person should be able to open the case on their own.

- Seals to case 	Airtight.   Seals and sealants must

be tested and approved by Conservation

(see No. 8 Testing Materials).

No.3 Specifications for Design of Showcases Contd.

Lighting	 	Either in a separate vented box at top of or

outside showcase.

Transformer(s) outside showcase and light box.

UV filtered Perspex or glass between light

and show case. This should be fitted with a trim or other dust proofing method to prevent dust falling into case below.

Diffuser sheet as required.

Access by separate panel outside the showcase.

Fibre optic lights. To be sealed at the back with “O” rings.    All access points for tails to be sealed.

Ventilation	           Light box always vented.

Rest of showcase only if specifically requested by Conservation.

Security	           Locks: suited in liaison with Facilities.

Light levels		To be advised by Conservation.

Zero UV in all areas.

Lux levels to be established depending on exhibits.

No direct daylight.

Light fittings	          	Low energy.

No direct heat.

Avoid  'bright' spots.

All must have electrical dimming.

Use of filters as required.

Separate electric lighting controls.

Monitoring	Some show cases may require the installation of a humidity and temperature sensor connected to the Museum's Building

Management System, or discrete stand-

alone electronic monitors.   All access

points must be sealed.

Access Port	Fitted to all showcases.    Standard brass

fitting with sealed cap (Conservation to

advise).

Environmental	Some showcases may require an

Control Facility	accessible space for material such as desiccants or adsorbents. An adequate gap or grille to be provided to allow effective conditioning of internal air.

HORNIMAN MUSEUM & GARDENS

COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES FOR DEVELOPMENT AND CONSTRUCTION OF EXHIBITIONS, DISPLAYS AND

GALLERY REFURBISHMENT

No.4 Showcase Construction Materials

Standard

High specification display case design to assist in achieving environmental standards.

Guidelines

Designers and contractors should work closely with Conservation, Facilities and Exhibition staff to detail showcase specification. It is not intended to give a detailed list of materials that can or cannot be used in a Museum context; the working practices described below are for guidance only.  Manufacturers frequently change specifications, 'improve' products or release new products onto the market. (see No. 8 Testing Materials).

All new or renovated showcases must be allowed to ventilate with the access point open to allow for off gassing (see No. 8 Off-gassing).

The problems of dust production within a display space should not be underestimated

	All metal components to be powder coated

		Any wood or composite board used in the construction

must be approved and tested by Conservation:  e.g.

softwoods or formaldehyde free MDF (Medite Exterior or

ZF Medite from a reliable source where the panels have

been stored according to Manufacturer’s instructions)

		Any wood or composite board used in the construction must be sealed with 3 coats of approved sealant such as Dacrylate or Shellac, or covered with Moistop®, all to manufacturer’s instructions, including areas to be painted. (see No.5 Showcase dressing materials)

	Laminated glass for glazing

	Interior joints sealed – with tested sealants.

	(see No. 8 Testing Materials)

No.4 Case Construction Materials contd.

Case construction may vary from one Gallery of the Museum to another, depending on the general climate control in that Gallery.

For example:

South Hall Anthropology Gallery and North Hall Natural History Gallery.

These are large high vaulted spaces with partial climate control with heated and humidified air. There is no cooling or dehumidification. Consequently the emphasis is placed on good showcase design for the protection of the exhibits.

In addition to the general considerations outlined above, attention would need to be paid to:

	airtight construction with good dust seals with air exchange rate to less than 0.1%

	provision for showcase conditioning such as Prosorb™, a buffering agent

Centenary Gallery, Music Gallery and Temporary Exhibition Gallery

These spaces have full air conditioning. General considerations as outlined above are applicable, but occasionally further provision may have to be made for sensitive specimens which require special conditions, e.g. use of a desiccant or adsorbent.

HORNIMAN MUSEUM & GARDENS

COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES FOR DEVELOPMENT AND CONSTRUCTION OF EXHIBITIONS, DISPLAYS AND GALLERY

REFURBISHMENT

No.5  Showcase Dressing Materials

Standard

Case dressing materials must provide a stable environment for objects

Guidelines

By their nature showcases are enclosed and therefore potentially destructive environments. They must provide a safe, secure, stable environment for objects which may be on display in the same space for some years. Display mounts/fittings must allow for easy access for future maintenance of displays. (See also No.8 Testing materials and Off -gassing periods)

Mount board/	e.g. Perspex®, glass and acid free

overlay panels:   	card,Formaldehyde free MDF (e.g. Medite Exterior or ZF Medite from a reliable source where the panels have been stored according to manufacturer’s instructions) must be coated with 3 coats of an approved sealant such as Dacrylate or Shellac, or covered with Moistop®, all to manufacturer’s instructions.

When more sensitive objects are present. Conservation may recommend tighter specifications.

Adhesives:	PVA based adhesives only if essential and passed by Conservation

Paints:		Must be water based emulsion paints with inert pigments and passed by Conservation.

As a rule however paints should not be used within showcases.

Oil based paints should not be used in Galleries or on display cases

No.5 Case Dressing Materials contd.

Fabrics:                   	All fabrics must be tested (See No.8 Testing Materials).  The Horniman and the British Museum have databases of previously tested and approved fabrics which can be consulted as a starting point. All fabrics still need to be tested,

particularly if it has been some time since the original test was done.

Graphic Panels:        	Foamex, resin coated paper, screened Perspex, all-rag museum board or other agreed materials

(inside cases &

inc. labels) 	Use of spray mounting adhesive to

be avoided. If used see No. 8 Testing Materials and off Gassing

Any heat seal film must be polyester based and approved by Conservation.

Wood			Only if tested and approved by

Conservation.

HORNIMAN MUSEUM & GARDENS

COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES FOR DEVELOPMENT AND

CONSTRUCTION OF EXHIBITIONS, DISPLAYS AND

GALLERY REFURBISHMENT

No.6    Open Displays

Standard

Every due regard and effort must be made to minimise and mitigate the threat to the objects.

Statutory Health & Safety Regulations must be observed.

Guidelines

An object selected for open display is under constant threat. It must be borne in mind that every time an object is cleaned of even a light film of dust, the surface suffers from wear and tear.  The problems of dust production within a display space should not be underestimated.

One of the major threats however is people and the security of the objects is a major issue for discussion with all Departments within the Museum regarding responsibilities, approaches and solutions.

The design must consider:

	protective 'roof' to mitigate dust falls.

- Ease of access for maintenance. e.g. can objects on high

be reached/lowered/cleaned without disturbing other

objects on display?

	A barrier between the object(s) and people; this could be

in the form of: distance/space, transparent 'wall' e.g.

Perspex, theatre netting, or a 'fence '. People must not be

able to reach/touch/pull/purloin any part of the object(s).

	Security measures in the form of alarms to be discussed

with Facilities (see also general security issues above).

	All objects must be 'secured' to display components.

	Plinths for floor mounted objects, if necessary with a

'skirting'  to stop 'kicks', and provide protection from floor cleaning activities

- If wood floor to exhibition space incorporate ‘dust

catchers’ such as dust mats at doorways 
HORNIMAN MUSEUM & GARDENS

COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES FOR DEVELOPMENT AND CONSTRUCTION OF EXHIBITIONS, DISPLAYS AND

GALLERY  REFURBISHMENT

No.7 Mounts and Fixings

Standard

Mounts and fixings must provide a safe and secure support for objects.

Guidelines

General :

Methods of fixing objects to the showcase fittings, such as individual mounts, should prevent the object suffering excessive vibration or wear.  It should not be possible for an object to slip from a shelf and suffer damage. The possible loading on a shelf or mount must be checked.

- All mounts/fixings must be approved by Conservation.

No mount/fixing must cut into objects. Conservation to advise on sensitive objects and areas of objects requiring special support.

No object must be in contact with painted or varnished surfaces (See No  5  Case Dressing Materials).

All materials used must be approved/tested by Conservation  (see  No. 8  Testing Materials & Off-gassing)

	Acceptable Materials	-Perspex, glass

Include:		           -Metal supports with heat seal films

as approved by Conservation.

-Polyethylene tubing may be used as

ties if necessary

- Other materials to be approved &

tested by Conservation ( see No.8

Testing Materials & Off-gassing)

HORNIMAN MUSEUM & GARDENS

COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES FOR DEVELOPMENT AND

CONSTRUCTION OF EXHIBITIONS, DISPLAYS AND

GALLERY  REFURBISHMENT

No. 8 Specification for Testing Materials

Standard

All materials proposed for use must be tested and approved.

Guidelines

Many of the materials available for Exhibitions, Displays and Gallery refurbishments give off acids and other vapours that can harm museum objects. The Horniman Museum has a strict regime of testing all materials proposed for use. This also applies to materials long established as suitable for museum use. Manufacturers are known to change their fabrication methods and there is no alternative but to test samples from the suppliers that the Designer/Contractor will be using.

Samples must be presented to Conservation at least 8 working weeks in advance. The test takes four weeks and the Conservation Laboratory has limited facilities for testing materials. Batches of 10 different materials can be tested at any one time.

Conservation will advise on Laboratories that can carry out any particularly specialised test not available at the Horniman. Such out-house costs must be borne by the Project budget.

	Information required:	-Name of manufacturer

-Name of  supplier

-Trade name of product

-Technical MSDS/COSHH data sheet

-At least 10g of material to be tested

	Off-Gassing

Enough time must be allowed for drying-off and off-gassing (e.g. solvents and water) from materials, particularly sealants, adhesives, paints and glazes and any building works such as plastering. The minimum time is stipulated below.  In case of multiple coatings, each coat must be allowed to off gas before applying further layers

No.8 Specification for Testing Materials contd.

Guidelines for off-gassing time BEFORE installation of objects

- 8-6 weeks		-plaster work

- 6 weeks		-solvent based paints

4-2 weeks	- use of adhesives            completed (including spray mounting)

	2 weeks		- glazing and painting completed

	2-3 weeks		- sealants

- 2-3 weeks                - water based paints

HORNIMAN MUSEUM & GARDENS

COLLECTIONS CONSERVATION & CARE

STANDARDS AND GUIDELINES FOR DEVELOPMENT AND

CONSTRUCTION OF EXHIBITIONS, DISPLAYS AND

GALLERY REFURBISHMENT

Appendices I - III:

Horniman Museum & Gardens Codes of Practice

I.	‘General Working Practices’. Document issued to Contractors on arrival on site. Available from   Facilities Manager Horniman Museum

	Pre-contract Document Site Rules’ available from Facilities/Exhibitions Manager, Horniman Museum.

III.	‘Horniman Museum & Gardens Guidelines for Handling Museum Objects’ available from Head of  Collections Conservation & Care Section, Horniman Museum